Personal GPS Tracker

USERS MANUAL

V6.4

Updated on 2010/06/28

1 Product Overview

This Tracker provides peace of mind by showing you with your mobile phone or PC the location of your targets via SMS or GPRS.

TRACKER has the following features

- Tracking via SMS or GPRS

- Current location report

- Tracking by time interval

- Two-way conversation

- SOS alert

- Geo-fencing control

- Low battery alert

- Speeding alert

- Four buttons for making phone call and/or sending message

- Removable and rechargeable internal battery

- Built-in speaker and microphone

- Alert when Track enter/exits GPS blind area

- Live Voice monitoring

- sleeping mode: when the geo-fence alert and over speed alert is closed , the sleeping mode is active. When the geo-fence or orver-speed alert is enabled , the sleeping mode is closed and gps is keeping on .

2 For Your Safety

Read these simple guidelines. Not following them may be dangerous or illegal. Read the full user manual for more information.

Switch on safely- Do not switch on Track when wireless phone use is prohibited or when it may cause interference or danger.

Switch off in Hospitals– Follow any restrictions. Switch Track off near medical equipment.

Switch off in Aircraft – Follow any restrictions. Wireless devices can cause interference in aircraft.

Switch off when refueling– Do not use Track when at a refueling point. Do not use near fuels or chemicals.

Water Resistance –Track is not water resistant. Keep it dry. Use waterproof bag if necessary.

3 Track Parameters& Panel Description

| Item | Specification |
| --- | --- |
| Charging Voltage | DC 5V/500mA (Micro USB port) |
| Internal Battery | Rechargeable and removable 830 mAh battery (3.7V) |
| Operating temperature | -20° to 55° C |
| Humidity | 5% to 95% Non-condensing |
| GSM module | GSM 900/1800/1900Mhz |
| GPS Chipset | latest GPS (AGPS) |
| GPS Sensitivity | -158Db |
| GPS Frequency | L1, 1575.42 MHz |
| C/A Code | 1.023 MHz chip rate |
| Channels | 20 channel all-in-view tracking |
| Position Accuracy | 10 meters, 2D RMS |
| Velocity Accuracy | 0.1 m/s |
| Time Accuracy | 1 us synchronized to GPS time |
| Default datum | WGS-84 |
| Reacquisition | 0.1 sec., average |
| Hot start | 1 sec., average |
| Warm start | 38 sec., average |
| Cold start | 42 sec., average |
| Altitude Limit | 18,000 meters (60,000 feet) max. |
| Velocity Limit | 515 meters/second (1000 knots) max. |
| Acceleration Limit | Less than 4g |
| Jerk Limit | 20 m/sec |
| Work time | 60 hours in sleep mode and 10 hours in normal mode |
| LED | 3 LED indicators showing power, GPS, GSM status and other operation status. |

Panel Description

4 Getting Started

4.1 Hardware and Accessories

- USB Power Cable

- User Manual

- Other document or software if required

4.2 Light and Button Functionality

TRACK has four buttons and three LED lights with three different colors to indicate the status of the unit. All LED is enabled when power on, and short click SOS key to change GSM & GPS LED enable / disable status.

| 3 led are flashing from left to right | The Sim card is not inserted or SIM card is not in normal |
| --- | --- |

| Red LED | Indicating power status |
| --- | --- |
| Off | Power is off / charging is complete/in sleeping mode |
| On | charging |
| Flashing(on for 0.01sec and off for 4.99 sec) | Low power |
| Flashing (on for 1 sec and off for 4 sec) | Tracker is out of sleeping mode |

| Green LED-indicating GPS status |  |
| --- | --- |
| Off | Device is not ready |
| flashing in 0.05 seconds and 4.95 seconds off | GPS fixed |
| Flashing(on for 1 second and off for 4 seconds) | GPS no fix / GPS power off |

| Blue LED-indicating GSM status |  |
| --- | --- |
| On | One call is coming in / one call is being made |
| Flashing (on for 0.05 second and off for 4.95 seconds) | TRACK is connected to GSM network |
| Flashing (on for 1 second and off for 4 seconds) | TRACK is not connected to the GSM network |

| Power Button | To turn on/off |
| --- | --- |
| SOS Button | Long press SOS button to send an alarm to location center and make a phone call and/or send an SMS to the preset phone number. Short press SOS button to change GSM & GPS LED enable / disable status. |
| 1 Button | Long press 1 button to make a phone call and/or send an SMS to the preset phone number. |
| 2 Button | Long press 2 button to make a phone call and/or send an SMS to the preset phone number. |
| 3 Button | Long press 3 button to make a phone call and/or send an SMS to the preset phone number. |

4.3 First Use

Ensure that Track has a working SIM installed

- Check that the SIM has not run out of credit

- Check that the SIM Lock code is turned off

Use some other mobile phone to confirm that the PIN code Can not been set ,and that it can dial out and receive telephone calls, and that it can connect to internet by GPRS.

- Charge the battery for at least 3 hours in power-off status using the wall charger or car charger. Red light is on during charging and is off when charging is complete.

4.4 Operation Instructions

- Power On/Off

- Power On:

Step 1. When it is off, press the power button

for 5 second to switch on the power.

Step 2. After power on, GSM LED will light on first (on 1 second and off 4 seconds for searching GSM network, or on 0.05 second and off 4.95 seconds after get GSM network).

Step 3. Some 10 seconds later, GPS LED starts to flash (on 1 second and off 4 seconds for searching GPS, or always on after GPS fixed). By then, the device is ready to work.

- Power Off:

When it is on, press the power button for 3 seconds to switch off the power; Power/GPS/GSM LED will shut off right now. By then, the device is turned off.

- SOS Emergency Help Function

When SOS button is long pressed, Track will send alarm to location center, and send a SMS message and make phone call for emergency help. The phone number can be set by the users.

When 1 2 3 button is long pressed, Track will make normal phone call. The phone number can be set by users. The phone number list can be set by SMS command.

- Conversation Function

Call In: One can make phone call to Track by simply dialing the SIM card number.

To receive the incoming phone call.

To refuse the incoming phone call.

Call Out: Track has four speed dial; you can long press preset 4 sets of phone

5 SMS command

The device only reply to command with valid password . It will neglect the  command iwth invalid pasword. Factory setting password is 000000。

6 GPRS Settings by SMS

Tracking via GPRS, you should set APN for Track

Ensure that your SIM card in Track supports GPRS connection prior to setting.

7 set APN

Command: W<password>,011,APN,APN Name,APN Password,APN DNS,APN Second DNS

Note: If no APN name, APN password, APN DNS and APN Second DNS required, input APN only.

Example: W000000,011,CMNET

TRACK will reply with an SMS (like ‘W01,011, Set device APN OK’) to confirm this setting.

（FOR example ：‘W01,011, Set device APN OK’）。

8 SMS command list

note: ****** is password，factory setting password: 000000.

| Description | Command | Remarks |
| --- | --- | --- |
| Get current location | W******,000 | Get current location of tracker |
| Change user’s password | W******,001,###### | ****** is old password ###### is new password max 6 characters. |
| Set interval for automatic timed report | W******,002,XXX | XXX is the interval in minute. If XXX=000 it will stop tracking |
| Set preset phone number for SOS button | W******,003,F,P,T | F=0, to turn off this fuction; F=1, only sending SMS; F=2, only calling preset phone number; F=3, both SMS and calling (default) P=1, set a authorized number for SOS button P=2, set a authorized number for 1 button P=3, set a authorized number for 2 button P=4, set a authorized number for 3 button T: Preset phone number. Max. 16 digits |
| Set over speed alert When tracker speeds higher than the preset value, it will send one over speed alert to the SOS preset number. | W******,005,XX | XX (the preset value of speed) =00 , to turn off this function =[01, 20] (unit: 10Km/h) For example, W000000,005,08, it will sent alert when it is over 80Km/h |
| Set Geo-fence alert When tracker moves out of preset scope, it will send one Geo-fence SMS to the SOS preset number. | W******,006,XX | XX ( preset distance to original place ) =00, to turn off this function =01, 30m =02, 50m =03, 100m =04, 200m =05, 300m =06, 500m =07, 1000m =08, 2000m |
| Extend Settings | W******,008,ABCDEFG | A=0, turn off the function of sending an SMS location report to the authorized phone number when it makes a call to tracker. A=1, turn on the function of sending an SMS location report to the authorized phone number when it makes a call to tracker. B=0, Google address B=1, location data complies with NMEA 0183 GPRMC protocol. For example: $GPRMC,072414.000,V,3114.3763,N, 12131.3255,E,0.00,0.00,050805,*00 C=0, turn off the function to automatically hang up an incoming call. C=1, turn on the function to automatically hang up an incoming call after 4 - 5 rings. D=0, Turn off the function of sending an SMS when tracker is turned on. D=1, Turn on the function of sending an SMS to SOS number when tracker is turned on. E=0, reserved E=1, tracker shuts down automatically when the power voltage lower than 3V. F=0, Turn off the alert when tracker enters GPS blind area. F=1, Turn on the alert when tracker enters GPS blind area. SMS is to be sent to SOS number G=0, 3 LED lights work normally G=1, 3 LED lights stop flashing when tracker working. (default value is: ABCDEFG=0100100 ) |

| Presetting by SMS for GPRS tracking (Ensure that your SIM card supports GPRS connection prior to setting) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Set ID for tracker | W******,010,ID |  |  | Tracker ID must not over 14 digits. |  |
| Set APN | W******,011,APN,APN Name,APN Password,APN DNS,APN Second DNS |  |  | If no APN name and password required, just insert APN only; APN defaulted as ‘CMNET’; APN + APN name + APN password + APN DNS + APN Second DNS not over 39 characters. |  |
| Set IP Address and Port | W******,012,IP, Port |  |  | IP: xxx.xxx.xxx.xxx Port: [1,65534] |  |
| Enable GPRS Tracking Function | W******,013,X |  |  | X=0, turn off GPRS tracking (default); X=1, enable TCP |  |
| Set Time Interval for Sending GPRS Packet | W******,014,XXXXX |  |  | XXXXX should be in five digitals and in unit of 10 seconds. XXXXX=00000, to turn off this function; XXXXX=00001~65535, time interval for sending GPRS packet and in unit of 10 seconds. |  |
| For more information regarding GPRS settings please refer to <GPRS Communication Protocol> |  |  |  |  |  |
| Set phone number for wiretapping |  | W******,030,T | T is the telephone number for wiretapping and max. 16 digits |  |  |
| Set time zone difference |  | W******,032,T | T=0, to turn off this function T=[-720, 720] to set time difference in minute to GMT. Default value is GMT For those ahead of GMT, just input the time difference in minute directly. For example, W000000,032,120 ‘-‘is required for those behind GMT. For example, W000000,032,-120. |  |  |
| Set character for SOS alert message |  | W******,033,P,Char | P is the button number. P=1, 2, or 3. P=1, SOS button P=2, 1 button P=3, 2 button P=4, 3 button Char is the character in SOS message and max 32 characters |  |  |
| AGPS |  | W******,043,IP,port,sms_num | it can be set by SMS command . set AGPS SUPL server by SMS |  |  |
| Set RTC |  | W******,044,YYYYMMDD,hhmmss |  |  |  |
| Restore factory setting |  | W******,048 | restore factory setting |  |  |
| Got the current location in globusgps format |  | W******,051 | Get the current tracker coordinates in globusgps format as following : <NavitelLoc>N55.83923°E037.45797°<N>2011-06-01<D>24.0000Km/h |  |  |
| Got location in Google link |  | W******,100 | Get location through google link http://maps.google.com/maps?f=q&hl=ru&q=55.751,37.616&ie=UTF8&z=16&iwloc=addr&om=1 , click directly the link to log into google map to view the location . |  |  |
| Got location in detailed address |  | W******,052 | Get current location in detailed address |  |  |
| Get version and serial number |  | W******,600 | To get version and serial number of current firmware |  |  |
| Get IMEI |  | W******,601 | To get IMEI |  |  |

9 Tracking on Map

Download Google Earth from http://earth.google.com/

Start the google earth

(For more in formation about Google Earth please refer to http://earth.google.com/) as the following picture shows:

Input the latitude and longitude that you receive from the tracker by SMS and click the search button

Google Earth will display the location for you.

Example:

When you receive:

Latitude=22 32 40.05N

Longitude=114 04 57.74E

Type as the following picture shows:

(Note: you should input the latitude and longitude as:22 32.4005N,114 04.5774E)

Now you can find the location of your tracker:

Or you can start the Internet Explorer and connect to http://maps.google.com for displaying the location.

Or you can use local map software on PDA or car navigation device to input the coordinates.

10 Troubleshooting

| Problem:Unit will not turn on |  |
| --- | --- |
| Possible Cause: | Resolution: |
| The power button is not pressed long enough | Press and hole the button for 3-5seconds |
| Battery needs charging | Recharge battery for 3 hours |
| Problem:Unit will not reply with SMS |  |
| Possible Cause: | Resolution: |
| Green LED is flashing(on for 1 second and off for 2 seconds) | Make Tracker connected to GSM network |
| GSM Network is slow | Wait for SMS.Some GSM networds slow down during peak time or when they have equipment problems. |
| Unit id sleeping | Cancel sleeping mode |
| Wrong password in your SMS or wrong SMS format | Write correct password or SMS format |
| The SIM in Tracker has run out of credit | Replace or top up the SIM card |
| Problem:Green LED is Flashing(on for 1 second and off for 2 seconds) |  |
| Possible Carse: | Resolution: |
| No GSM signal | Check with a mobile phone to see if there is a signal in the area or try to call the unit to see if you hear a ring tone. |
| NO SIM card | Insert a working SIM card.Check in phone that the SIM can send SMS message. |
| SIM card ha expired | Check in phone that the SIM can send SMS message.Replace SIM card if needed. |
| SIM has PIN code set | Remove PIN code by inseting SIM in you phone and deleting the code |
| SIM is warped or damaged | Inspect SIM,clean the contacts.If re-inserting does not help try another to see if it will work. |
| Roaming not enabled | If you are in a different country your SIM account must have roaming enabled. |
| Battery is low | Recharge the unit and the GSM will start working |
| Problem:Blue LED is Flashing (on for 1 second and off for 2 seconds)or the SMS received strats with ‘Last…’ |  |
| Possible Cause: | Resolution: |
| Unit does not have clear view of the sky | Move the unit to a location where the sky is visible.Tall buildings,trees,heavy rain,can cause problems with the GPS reception. |
| Bad GPS reception | Place the front side of Tracker towards sky |
| Battery is low | Recharge the unit and the GPS will start working |

## Message return format: W**,###

###: Prompt number

**: Error Hint

00: Get OK

01: Setting OK

02: Setting Fail

03: Message format Error

04: Password Error

05: Prompt Number Error

06: Modify Password Error

07: Parameter data Error

08: No GPS data

09: Not Support Protocol

10: Over Speed

11: Beyond the Field

12: Invalid GPS data

13: Open Port Error

14: Over time

15: Refuse

16: Get GPS Error

17: Momery arrange Error

18: Auto Alert

19: GPRS OK