Vehicle GPS Tracker

User manual

V7.2

Updated on 2013/04/22

This Tracker provides peace of mind by showing you with your mobile phone or PC the location of your targets via SMS or GPRS.

TRACKER has the following Main features

- Real-time Tracking via SMS or GPRS

- Support GPS and LBS

- Tracking by time interval

- SOS alert

- Geo-fencing control

- Over-speed alert

- Alert when Track enter/exits GPS blind area

- Live voice monitoring

- Vibration alert

- Advanced Static drift

- Anti-theft function

- Support TTS function

- Support  MIC , SOS , SPEAKER , IMMBOLIZATION , ACC .

- Mileage report

- Wide operating voltage form 5.5V to 40V

- Parameters& Panel Description

| Item | Specification |
| --- | --- |
| Voltage | 5.5-40V |
| Operating temperature | -20° to 65° C |
| Humidity | 5% to 95% Non-condensing |
| GSM module | MTK6252 |
| GSM Frequency | GSM 900/1800/1900Mhz/850mhz 4 bands |
| GPS Chipset | MTK3326 |
| GPS Sensitivity | -158Db |
| GPS Frequency | L1, 1575.42 MHz |
| GPS Channels | 20 channel all-in-view tracking |
| Position Accuracy | 10 meters, 2D RMS |
| Velocity Accuracy | 0.1 m/s |
| Time Accuracy | 1 us synchronized to GPS time |
| Default datum | WGS-84 |
| Reacquisition | 0.1 sec., average |
| Hot start | 1 sec., average |
| Warm start | 38 sec., average |
| Cold start | 42 sec., average |
| Altitude Limit | 18,000 meters (60,000 feet) max. |
| Velocity Limit | 515 meters/second (1000 knots) max. |
| Acceleration Limit | Less than 4g |
| Jerk Limit | 20 m/sec |
| LED | 3 LED indicators showing power, GPS, GSM status and other operation status. |
| G-sensor | ±2g |
| SIZE | 85*46*10mm |

Panel Description

Back cover printing :

2. Product Overview

  - check part list

before starting , check all the following items have been included with your I86 , if anything is missing , please contact your supplier

| Items picture | Name | Description |
| --- | --- | --- |
|  | Extended Power wire with length of 1.5M | the red indicate the positive , the black wire indicate negative (GND). The wire is with Fuse included ,which is to protect the device being burned . |
|  | Extended ACC wire with length of 1.5M | Green wire indicate ACC , white wire is for GND .( GND is not necessary to connect ) |
|  | Panic button with length of 1.5M |  |
|  | User manue |  |

  - Interface Definition

The I86 has 10 pin interface connectors . it contains the connector for power , SOS, ACC, Speaker, Microphone, relay . the sequence and definition of the 10pin are shown in the following figure:

3.  Getting started

3.1 open the back cover and insert SIM card

3.2 wire connection

4. LED indication instruction :

Device has Three LED lights with three different colors to indicate the status of the unit.

Red led-indication power status

| Green led-indicating gps status |  |
| --- | --- |
| Off | Device is not ready |
| flashing in 0.05 seconds and 4.95 seconds off | GPS fixed |
| Flashing(on for 1 second and off for 4 seconds) | GPS not fixed |

| Blue led-indicating GSM status |  |
| --- | --- |
| On | when call is coming in / call is being made/GPRS transmitting |
| Flashing (on for 0.05 second and off for 4.95 seconds) | Track is connected to GSM network |
| Flashing (on for 1 second and off for 4 seconds) | Track is not connected to GSM network |

Note: when the device is on, if it is not insert SIM card or abnormal SIM card, the three LED would be continuously flashing in sequence

5 SMS command

The device only reply to command with valid password . Factory setting password is 000000.

5.1 Basic SMS command

5.1.1 current location required command :

Description：if u need to know the current location of the tracker, kindly send the following

Command: W<password>, <000>

Note: default password is 000000.

For example:

Sending SMS: W000000,000

Receiving SMS: W00,000,$GPRMC,034522,662,A,2232,3126,N,11359,3061,E,000.0,000.0,241209,,,A*6E

5.1.2 authorize SOS phone NO.（SOS key is an optional accessory)

Description：binding the phone No.for SOS. When press the SOS key, it will send SOS message alert to the binded phone No.

Command: W<password>,003,F,P, T

Note:

(a) F =0, to turn off this function

=1, only sending SMS

=2, only calling the preset phone No.

=3, Both SMS and Calling

P =1, set authorized No. for SOS button

T, preset phone No.

(b) When long press the SOS key, it can send message to the preset phone

Example:

Sending SMS: W000000,003,1,1,123456789

Receiving SMS: W01,003,Set phone number OK

Remarks: when long press SOS key, SOS message will be sent to“123456789”。

5.2  GPRS Settings by SMS

Tracking via GPRS, you should set APN for Tracker

Ensure that your SIM card in Track supports GPRS connection prior to setting.

5.2.1 Setting APN

Command: W<password>,011,<APN>,<APN user name>,<APN password>,<APN DNS>,<APN second DNS>

Note: If no APN name, APN password, APN DNS and APN Second DNS required, input APN only.

Example: W000000,011,CMNET

TRACK will reply with an SMS (like ‘W01,011, Set device APN OK’) to confirm this setting.

5.3 Setting for Anti-theft function

5.3.1 To enable this function

Sending SMS to the preset No. to binded the sos key is to enable the anti-theft function（pls refer to the setting of preset phone No. for SOS button)

5.3.2 Setup defense

After successfully sending”SF” to device to setup defense, the device will enter into the security status with one minute delay. Once the vehicle is moved or vibrated illegally, the message alarm will be sent to the authorized No.

5.3.3 Cancel defense

After successfully sending “CF” to device to cancel defense, the device will be back to normal status.

5.3.4 Get position

sending command ”DW” to device, the tracker will reply the current detailed location

5.3.5 Get position in coordinates

Sending command” DWJW” to device, the tracker will reply with the position data(coordinates and cell information)  .

6 SMS command list

Note:  ****** is password，factory setting password: 000000.

| Description | Command | Remark |
| --- | --- | --- |
| Get current location(NMEA) | W******,000 | Get current location of tracker in NMEA format |
| Get current location(Google Earth) | W******,051 | Get current location in Google Earth format |
| Get location in detailed address | W******,052 | Get current location in detailed address |
| Change user password | W******,001,###### | ****** is old password ###### is new password |
| Set preset phone number for SOS button | W******,003,F,P,T | F=0, to turn off this function; F=1, only sending SMS; F=2, only calling preset phone number; F=3, both SMS and calling (default) P=1, set a authorized number for SOS Button T: Preset phone number. Max. 16 digits |
| Adjust G-sensor sensitivity | W******,017,XX | XX=01-15，default is 02 01 is for the maximum,15 is for the minimum |

| Presetting by SMS for GPRS tracking (Ensure that your SIM card supports GPRS connection prior to setting) |  |  |
| --- | --- | --- |
| Set APN | W******,011,APN,APN Name,APN Password,APN DNS,APN Second DNS | If no APN name and password required, just insert APN only; |
| Set character for SOS alert message | W******,033, P,Char | P=1, SOS key Char is the character in SOS message and max 32 characters |
| Restore factory setting | W******,048 | Restore factory setting |
| Got location in Google link | W******,100 | Got location through Google link |
| Get version and serial number | W******,600 | To get version and serial Number of current firmware |
| Get IMEI | W******,601 | To get IMEI |
| Immbolization | W000000,911,1(0) | “1” indicate to enable engine cut , “0” indicate to disenable engine cut. |
| Setup defense | SF | Binding SOS key to enter into security mode. |
| Cancel defense | CF | Binding SOS key to disenable the security mode. |
| Get position | DW | Binding SOS key to get detailed current location |

Note 1

## Message return format: W**,###

**	Error code

###	Prompt No.

Error code description :

00: Get ok

01: Setting ok

02: Setting fail

03: Message format error

04: Password error

05: Prompt No. error

06: Modify password error

07: Parameter data error

08: No GPS data

09: Not support protocol

10: over-speed

11: Beyond the field

12: GPS invalid data

13: Open port error

14: Over-time

15: Refuse

16: Get GPS data error

17: Memory arrange error

18: Auto alert

19: GPRS ok